# Scope

This report details the calculations and justifications for starting circuit fuse selections for various machines at Newmont’s Cadia site.

# Background

Starting circuits on heavy machinery have been identified as a fire risk as they have typically lacked protection upon delivery by the OEM. GBEV was tasked with creating a tool to calculate the appropriate fuse rating for safeguarding the starting system during a short circuit event on various machines using a fact-based approach.

# Results

Table 1: Fusing Recommendations

| Make | Model | Suggested Fuse Size (A) | Cranking Time (s) | Crankinggui Current (A) | Time value (s) | Littelfuse PN | Fuse Holder PN | Parallel configuration (if required) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bell | B45E | 300 | 5 | 500 | 10 | 0298250.ZXEH_LT | GB161280 | N/A |
| Caterpillar | 777 (07) | 300 | 5 | 1200 | 5 | 0298500.ZXH | GB161280 | 2x 300A |
| Caterpillar | D10T | 500 | 5 | 1000 | 10 | 0298500.ZXH | GB161280 | N/A |
| Komatsu | 155 | 500 | 5 | 1000 | 10 | 0298500.ZXH | GB161280 | N/A |
| Komatsu | 375 | 500 | 5 | 1000 | 10 | 0298500.ZXH | GB161280 | N/A |
| Hitachi | 120T | 500 | 5 | 1000 | 10 | 0298500.ZXH | GB161280 | N/A |
| Caterpillar | 14M | 500 | 5 | 1000 | 10 | 0298500.ZXH | GB161280 | N/A |
| Caterpillar | 793F | 300 | 5 | 1200 | 5 | 0298500.ZXH | GB161280 | 2x 300A* |
| Caterpillar | 992K | 500 | 6 | 1000 | 10 | 0298500.ZXH | GB161280 | N/A |
| Caterpillar | D11 | 500 | 5 | 1000 | 10 | 0298500.ZXH | GB161280 | N/A |
| Hitachi | 69T | 500 | 5 | 1000 | 10 | 0298500.ZXH | GB161280 | N/A |

*(600A peak through each fuse = 1200A in total)

# Next steps

GBEV will continue to expand the functionality of the Fuse Selection Tool by adding more machines, components, and fuse options. Future iterations of the tool will be mobile app-based to allow for quicker assessment and ordering of required parts.

# Methodology

Once the machine model is selected, the fuse tool uses a comprehensive library to generate results with fusing information. This library includes measured data points on the machines, types of battery cables and their properties, available fuses suitable for short circuit applications, and details on popular alternators and starter motors along with their electrical specifications.

The tool performs calculations to ensure that the existing cable can handle the continuous and peak current requirements and that the voltage drop is within operational limits.

The tool provides the minimum battery voltage required for the electrical system to operate. If this minimum voltage cannot be supplied, it suggests replacing the relevant component or cable so that the machine operates within its electrical boundary conditions.

Finally, the tool displays the closest fuse match with the safety factor, relevant part number, and manufacturer information.

# Sample Calculations

To better understand the logic behind the Fuse Selection Tool’s process, the following section details two calculation scenarios from Table 1; a single fuse calculation and a parallel fuse calculation.

## Single Fuse Configuration

The following tables have been extracted from the Fuse Selection Tool. This example illustrates the calculations and logic statements for a single fuse configuration.

### User selection

Table 2: User Selection

| Category | Parameter | Values |
| --- | --- | --- |
| Fuse | Safety current factor (%) | 25 |
| Machine | Model | D10T |

Table 2 has two drop-down options:

- Fuse: The user can select either a 25 or 50% fuse rating safety current factor. In this example, the user has chosen 25%.

- Machine: The user can select the model of the machine from the list of models from the machine library. The user chose a D10T Dozer for this example.

### Boundary Conditions

Table 3: Boundary Conditions

| Category | Parameter | Values |
| --- | --- | --- |
| System | Minimum battery voltage for a 24V system (V) | 16.48 |
|  | Minimum battery voltage received by the starter motor (V) | 16 |
| Starter Motor | Output/Mechanical power (kW) | 9 |
|  | Starter motor efficiency during optimum conditions (%) | 56.25 |
|  | Peak current draw cut-off value (A) | 1000 |
|  | Maximum cranking time based on cables peak current capability (s) | 5 |

Table 3 extracts the electrical boundary conditions for the model D10T Dozer:

- System: The minimum voltage required for the machine to be operational with a 3% voltage drop in the cable from the battery to the starter motor.

- Starter Motor: Depending on the model of the starter motor used, specific cranking parameters are determined which serve as reference calculations for the cable and fuse.

### Measured Parameters & Electrical System Present

Table 4: Results for Measure Parameters & Electrical System

| Category | Parameter | Values |
| --- | --- | --- |
| Machine | Manufacturer | Caterpillar |
| Battery | What is the measured battery voltage during cranking (V)? | 16 |
|  | What is the measured cranking time (s)? | 5 |
|  | Should the battery be replaced due to low voltage or longer cranking time? | NO |
| Starter Motor | Starter Motor Peak/Cranking Current (A) | 500 |
|  | Is the cranking current less than the max value for the machine? | YES |
|  | Should the starter motor be replaced due to high inrush demand? | NO |
| Alternator | Alternator Continuous current (A) | 95 |
| Cable | Cable Type | Narva |
|  | Present Cable Size (mm2) | 70 |
|  | Operating Temperature (°C) | 60 |
|  | Cable continuous current (A) | 375 |
|  | Can the Present cable handle the continuous current demand? | YES |
|  | K factor (for copper) | 150 |
|  | Peak/short circuit current carrying capability (A) (time <= 5 s) | 4696.00 |
|  | Can the present cable handle the in-rush current demand? | YES |
|  | I2t calculated for allowed in-rush current (A) | 5,000,000 |
|  | Cable resistance (W/km) | 0.327 |
|  | Cable Length (m) | 6 |
|  | Voltage drop (%) | 9.2 |

- Machine: The relevant Manufacturer’s name is extracted for the D10T Dozer.

- Battery: The cranking voltage and time are measured, and the tool suggests if the battery needs to be replaced after comparing the values against the boundary conditions in Table 3. The battery may have a poor state of health that might cause the machine to be inoperable.

- Starter Motor: The tool takes the pre-measured cranking current for the starter motor and compares it to the worse case values in Table 3 to determine if the machine is cranking above the recommended value. If the current is higher than recommended, it suggests replacing the starter motor, as this could indicate low efficiency or the end of its useful life. Otherwise, the tool uses these boundary conditions.

- Alternator: The alternator's continuous current is determined from the machine data sheet and defines the continuous current demand from the machine's electrical system. This value is used in cable and fuse calculations.

- Cable: All relevant electrical characteristics of the battery cable are either directly measured from the machine or extracted from the typical cable used in heavy machinery. The tool then indicates if the existing cable can handle the continuous current and peak current demands.

### Fusing results

Table 5: Fusing Results

| Results for what is recommended based on the Time Current Characteristics graph |  |  |
| --- | --- | --- |
| Category | Enter Value | Values |
| Fuse | Current rating (A) based on continuous current demand. | 118.75 |
|  | Closet match for the fusing rating (A) | 125 |
|  | Is the fuse size available? If not, what is the next fuse size available? | 300 |
|  | How long can it handle the peak current (s)? | 0.75 |
|  | Can the Fuse handle the Peak current for the required cranking time? | NO |
|  | Which is the next fuse size that satisfies the requirement? | 392.5 |
|  | Closet match for the fusing rating (A) | 400 |
|  | How long can it handle the peak current (s)? | 2 |
|  | Can the Fuse handle the Peak current for the required cranking time? | NO |
|  | Does the new fuse rating protect the cable? | NO |
|  | Which is the next fuse size that satisfies the requirement? | 500 |
|  | How long can it handle the peak current (s)? | 10 |
|  | Can the Fuse handle the Peak current for the required cranking time? | YES |
|  | How much does the continuous current in the fuse de-rate for the operating temperature (A)? | 290 |
|  | Does the new fuse rating protect the cable? | YES |
|  | Do fuses need to be in parallel? | NO |
|  | Manufacturer | Little Fuse |
|  | Manufacturer's Part # | 298500.ZXH |
|  | Website link | https://www.littelfuse.com/products/fuses/automotive-passenger-car/high-current-fuses/298.aspx |
|  | GB Fuse holder part # | GB161280 |

Table 5 displays fusing results based on Table 2, Table 3, Table 4 and the Time -Current characteristics graph from the fuse manufacturer (see attached datasheet).

- The fuse rating is first calculated for continuous rating demand and then matches the closest fuse rating available verified database. If the suggested is not available in the database, it suggests the next available fuse rating.

- The tool then checks if the new fuse can handle the peak current demand for the maximum cranking time. If it doesn’t satisfy the condition, the tool continues to look for the next fuse rating until the conditions are satisfied.

- The tool then checks if the fuse can protect the cable.

- The tool finally recommends the appropriate fuse type, make and model, provides a link to the manufacturer’s website, and includes the GB fuse holder part number.

### Parallel Fuse Configuration

This example illustrates the calculations and logic statements for a parallel fuse configuration, which is required when the fusing requirements of the machine do not sufficiently match the existing fuse library.

### User selection

Table 6: User Selection - Parallel

| Category | Parameter | Values |
| --- | --- | --- |
| Fuse | Safety current factor (%) | 25 |
| Machine | Model | 777 (07) |

Table 6 has two drop-down options,

- Fuse: The user can select either 25 or 50% of the fuse rating Safety current factor. In this example, the user has chosen 25%.

- Machine: The user can select the model of the machine from the list of models from the machine library. The user chose a 777 (07) truck for this example.

### Boundary Conditions

Table 7: Boundary Conditions - Parallel

| Category | Parameter | Values |
| --- | --- | --- |
| System | Minimum battery voltage for a 24V system (V) | 16.48 |
|  | Minimum battery voltage received by the starter motor (V) | 16 |
| Starter Motor | Output/Mechanical power (kW) | 9 |
|  | Starter motor efficiency during optimum conditions (%) | 56.25 |
|  | Peak current draw cut-off value (A) | 1000 |
|  | Maximum cranking time based on cables peak current capability (s) | 5 |

Table 7 extracts the electrical boundary conditions for the model 777 (07) truck.

- System: The minimum voltage required for the machine to be operational with a 3% voltage drop in the cable from the battery to the starter motor.

- Starter Motor: Depending on the model of the starter motor used, specific cranking parameters are determined which serve as reference calculations for the cable and fuse.

### Measured parameters & Electrical system present.

Table 8: Results for Measured parameters / Electrical system present – Parallel.

| Results for what is measured / Present |  |  |
| --- | --- | --- |
| Category | Parameter | Values |
| Machine | Manufacturer | Caterpillar |
| Battery | What is the measured battery voltage during cranking (V)? | 18 |
|  | What is the measured cranking time (s)? | 5 |
|  | Should the battery be replaced due to low voltage or longer cranking time? | NO |
| Starter Motor | Starter Motor Peak/Cranking Current (A) | 1200 |
|  | Is the cranking current less than the max value for the machine? | NO |
|  | Should the starter motor be replaced due to high inrush demand? | YES |
| Alternator | Alternator Continuous current (A) | 150 |
| Cable | Cable Type | Weldflex |
|  | Present Cable Size (mm2) | 95 |
|  | Operating Temperature (°C) | 40 |
|  | Cable continuous current (A) | 375 |
|  | Can the Present cable handle the continuous current demand? | YES |
|  | K factor (for copper) | 150 |
|  | Peak/short circuit current carrying capability (A) (time <= 5 s) | 6373 |
|  | Can the present cable handle the in-rush current demand? | YES |
|  | I2t calculated for allowed in-rush current (A) | 5000000 |
|  | Cable resistance (W/km) | 0.236 |
|  | Cable Length (m) | 12 |
|  | Voltage drop(%) | 11.8 |

Table 8 extracts the relevant electrical specifications of the components used in the 777 (07) truck and the measured parameters.

- Machine: The relevant Manufacturer’s name is extracted for the D10T Dozer.

- Battery: The cranking voltage and time are measured, and the tool suggests if the battery needs to be replaced after comparing the values against the boundary conditions in Table 3. The battery may have a poor state of health that might cause the machine to be inoperable.

- Starter Motor: The tool takes the pre-measured cranking current for the starter motor and compares it to the worse case values in Table 3 to determine if the machine is cranking above the recommended value. If the current is higher than recommended, it suggests replacing the starter motor, as this could indicate low efficiency or the end of its useful life. Otherwise, the tool uses these boundary conditions.

- Alternator: The alternator's continuous current is determined from the machine data sheet and defines the continuous current demand from the machine's electrical system. This value is used in cable and fuse calculations.

- Cable: All relevant electrical characteristics of the battery cable are either directly measured from the machine or extracted from the typical cable used in heavy machinery. The tool then indicates if the existing cable can handle the continuous current and peak current demands.

### Fusing results

Table 9: Fusing Results - Parallel

| Results for what is recommended based on the TIME-CURRENT CHARACTERISTIC CURVES |  |  |
| --- | --- | --- |
| Category | Enter Value | Values |
| Fuse | The current rating (A) based on continuous current demand. | 187.5 |
|  | Closet match for the fusing rating (A) | 175 |
|  | Is the fuse size available? If not, what is the next fuse size available? | 300 |
|  | How long can it handle the peak current (s)? | 0.75 |
|  | Can the Fuse handle the Peak current for the required cranking time? | NO |
|  | Does the new fuse rating protect the cable? | YES |
|  | Do fuses need to be in parallel? | YES |
|  | How many fuses need to be in parallel? | 2 |
|  | How much does the total current de-rate (%)? | 10 |
|  | How much current will the starter motor draw from the battery to maintain the cranking current demand? | 1112 |
|  | Can the cable handle the new demand? | YES |
|  | How much will the total current derate when fuses are in parallel? | 1000.8 |
|  | How much current will flow through each fuse in parallel? | 500.4 |
|  | Manufacturer | Little Fuse |
|  | Manufacturer's Part # | 298300.ZXH |
|  | Website link | https://www.littelfuse.com/products/fuses/automotive-passenger-car/high-current-fuses/298.aspx |
|  | GB Fuse holder part # | GB161280 |

Table 9 displays fusing results based on Table 6, Table 7, Table 8 and the Time -Current characteristics graph (see attached data sheet)

- The fuse rating is first calculated for continuous rating demand and then matches the closest fuse rating available in the market. If the suggested is not available in the library database, it suggests the next available fuse rating.

- The tool then checks if the fuse can protect the cable.

- It checks if its fuse can handle the peak current demand for the maximum cranking time. If it doesn’t satisfy the condition, the tool then checks if a parallel configuration will meet the requirements.

- The tool then checks if the cable can handle the recalculated peak demand from the battery when the derating factor is considered for fuses in parallel and ensures the starter motor receives the maximum cranking amps from the battery.

- The tool then recommends the appropriate fuse type, make and model, provides a link to the manufacturer’s website, and includes the GB fuse holder part number.